2. Chebyshev IIR Filter

2.1. Design and Performance Evaluation of a Chebyshev Type-I IIR Low-Pass Filter using MATLAB

Program(MATLAB Code)

clc; clear;

Wp=0.4; Ws=0.55; Rp=1; As=40;

[N,Wn]=cheb1ord(Wp,Ws,Rp,As);

[b,a]=cheby1(N,Rp,Wn);

fprintf('Filter Order = %d\n',N);

fprintf('Cutoff Frequency = %.2f * pi rad/sample\n',Wn);

if all(abs(roots(a))<1)

disp('System is STABLE');

else

disp('System is UNSTABLE');

end

% -------- GRAPHS --------

figure; freqz(b,a);

% Magnitude + Phase

figure; impz(b,a);               % Impulse Response

figure; grpdelay(b,a);

% Group Delay

figure; zplane(b,a);             % Pole-Zero

Output:

Filter Order = 6
Cutoff Frequency = 0.40 * pi rad/sample
System is STABLE

Fig 1

Fig 2

Fig3

Fig 4

Result:

1. The Chebyshev Type-I IIR low-pass filter was successfully designed, and its magnitude response, phase response, impulse response, and group delay were analyzed.

2. The impulse response and group delay characteristics were obtained and found to satisfy the given specifications.

3. The pole-zero plot confirmed that all poles lie inside the unit circle, verifying the stability and proper performance of the filter.

4. The minimum filter order and cutoff frequency were successfully determined and found to satisfy the given design specifications.