Spectral Estimation of 5G Signals

Simulate the spectral estimation of a received OFDM signal using both Welch and AR methods. (25) (BL5) (WK4) Given Data: OFDM Subcarriers = 256  Sampling Frequency = 30.72 MHz  Tasks: Generate the OFDM signal.  Estimate spectrum using Welch method. Estimate spectrum using AR method. Compare spectral estimates.

Program (MATLAB Code)

clc;

clear;

close all;

%% SPECTRAL ESTIMATION OF 5G OFDM SIGNAL

% Given:

% OFDM Subcarriers = 256

% Sampling Frequency = 30.72 MHz

%% 1. GIVEN PARAMETERS

Nsc = 256;              % Number of OFDM subcarriers

Fs = 30.72e6;           % Sampling frequency

Nfft = 256;             % OFDM IFFT size

Nsym = 100;             % Number of OFDM symbols

CP = 32;                % Cyclic prefix length

SNR = 20;               % SNR in dB

fprintf('Number of subcarriers = %d\n',Nsc);

fprintf('Sampling frequency    = %.2f MHz\n',Fs/1e6);

fprintf('Number of OFDM symbols = %d\n',Nsym);

fprintf('Cyclic prefix length  = %d samples\n\n',CP);

%% 2. GENERATE RANDOM QPSK DATA

data = randi([0 3],Nsc,Nsym);

% QPSK modulation

qpsk = exp(1j*(pi/4 + pi/2*data));

%% 3. GENERATE OFDM SIGNAL USING IFFT

ofdm_symbols = ifft(qpsk,Nfft,1);

%% 4. ADD CYCLIC PREFIX

cyclic_prefix = ofdm_symbols(end-CP+1:end,:);

ofdm_with_cp = [cyclic_prefix;

ofdm_symbols];

%% 5. CONVERT INTO SERIAL SIGNAL

ofdm_signal = ofdm_with_cp(:);

% Normalize signal

ofdm_signal = ofdm_signal ./ sqrt(mean(abs(ofdm_signal).^2));

%% 6. ADD AWGN NOISE WITHOUT AWGN FUNCTION

signal_power = mean(abs(ofdm_signal).^2);

noise_power = signal_power/(10^(SNR/10));

noise = sqrt(noise_power/2) * ...

(randn(size(ofdm_signal)) + ...

1j*randn(size(ofdm_signal)));

received_signal = ofdm_signal + noise;

fprintf('SNR = %d dB\n',SNR);

fprintf('Total samples = %d\n\n',length(received_signal));

%% 7. TIME DOMAIN OFDM SIGNAL

t = (0:length(received_signal)-1)/Fs;

figure;

plot(t*1e6,real(received_signal));

xlabel('Time (\mus)');

ylabel('Amplitude');

title('Received OFDM Signal');

grid on;

xlim([0 20]);

%% 8. WELCH METHOD

% Parameters for Welch method

segment_length = 1024;

overlap = 512;

NFFT_Welch = 4096;

% Hamming window generated manually

n = (0:segment_length-1)';

window = 0.54 - 0.46*cos(2*pi*n/(segment_length-1));

% Number of segments

step = segment_length - overlap;

num_segments = floor((length(received_signal)-overlap)/step);

% Initialize PSD

Pwelch = zeros(NFFT_Welch,1);

%% 9. CALCULATE WELCH PSD

for k = 1:num_segments

start_index = (k-1)*step + 1;

end_index = start_index + segment_length - 1;

segment = received_signal(start_index:end_index);

% Remove mean

segment = segment - mean(segment);

% Apply Hamming window

segment = segment .* window;

% FFT

X = fft(segment,NFFT_Welch);

% Periodogram

P = abs(X).^2;

% Average

Pwelch = Pwelch + P;

end

% Average all segments

Pwelch = Pwelch / num_segments;

% Shift zero frequency to center

Pwelch = fftshift(Pwelch);

% Frequency axis

f_welch = (-NFFT_Welch/2:NFFT_Welch/2-1) ...

* Fs/NFFT_Welch;

% Convert to dB

Pwelch_dB = 10*log10(Pwelch + eps);

%% 10. PLOT WELCH SPECTRUM

figure;

plot(f_welch/1e6,Pwelch_dB,'LineWidth',1.2);

xlabel('Frequency (MHz)');

ylabel('Power (dB)');

title('OFDM Spectrum Using Welch Method');

grid on;

xlim([-Fs/2 Fs/2]/1e6);

%% 11. AR SPECTRAL ESTIMATION

% AR model order

AR_order = 20;

% Use real part for AR estimation

% This avoids toolbox-dependent AR functions.

x = real(received_signal);

x = x - mean(x);

%% 12. CALCULATE AUTOCORRELATION

maxlag = AR_order;

r = zeros(AR_order+1,1);

for k = 0:AR_order

r(k+1) = sum(x(1:end-k).*x(1+k:end)) / length(x);

end

%% 13. LEVINSON-DURBIN AR ESTIMATION

a = zeros(AR_order+1,1);

a(1) = 1;

E = r(1);

for m = 1:AR_order

numerator = r(m+1);

for j = 1:m-1

numerator = numerator + a(j+1)*r(m-j+1);

end

K = -numerator/E;

a_old = a;

a(m+1) = K;

for j = 1:m-1

a(j+1) = a_old(j+1) + K*a_old(m-j+1);

end

E = E*(1-K^2);

end

%% 14. AR POWER SPECTRUM

NFFT_AR = 4096;

f_ar = (-NFFT_AR/2:NFFT_AR/2-1)*Fs/NFFT_AR;

P_AR = zeros(1,NFFT_AR);

for k = 1:NFFT_AR

f = f_ar(k);

z = exp(-1j*2*pi*f/Fs);

denominator = 0;

for m = 0:AR_order

denominator = denominator + a(m+1)*z^m;

end

P_AR(k) = E/(abs(denominator)^2);

end

% Convert AR spectrum to dB

P_AR_dB = 10*log10(abs(P_AR)+eps);

%% 15. PLOT AR SPECTRUM

figure;

plot(f_ar/1e6,P_AR_dB,'LineWidth',1.2);

xlabel('Frequency (MHz)');

ylabel('Power (dB)');

title(['OFDM Spectrum Using AR Method, Order = ', ...

num2str(AR_order)]);

grid on;

xlim([-Fs/2 Fs/2]/1e6);

%% 16. NORMALIZE BOTH SPECTRA FOR COMPARISON

Pwelch_norm = Pwelch_dB - max(Pwelch_dB);

P_AR_norm = P_AR_dB - max(P_AR_dB);

%% 17. COMPARE WELCH AND AR METHODS

figure;

plot(f_welch/1e6,Pwelch_norm,'LineWidth',1.3);

hold on;

plot(f_ar/1e6,P_AR_norm,'--','LineWidth',1.3);

xlabel('Frequency (MHz)');

ylabel('Normalized Power (dB)');

title('Comparison of Welch and AR Spectral Estimates');

legend('Welch Method','AR Method');

grid on;

xlim([-Fs/2 Fs/2]/1e6);

%% 18. FREQUENCY RESOLUTION

resolution_welch = Fs/NFFT_Welch;

resolution_ar = Fs/NFFT_AR;

fprintf('Welch FFT size = %d\n',NFFT_Welch);

fprintf('Welch frequency resolution = %.2f kHz\n', ...

resolution_welch/1000);

fprintf('AR frequency grid spacing = %.2f kHz\n', ...

resolution_ar/1000);

fprintf('\n');

%% 19. FINAL COMPARISON

fprintf('\nWELCH METHOD:\n');

fprintf('1. Non-parametric spectral estimation\n');

fprintf('2. Gives a smooth spectrum\n');

fprintf('3. Reduces spectral variance by averaging\n');

fprintf('4. Suitable for noisy OFDM signals\n');

fprintf('\nAR METHOD:\n');

fprintf('1. Parametric spectral estimation\n');

fprintf('2. Uses an AR model\n');

fprintf('3. Can provide sharper spectral peaks\n');

fprintf('4. Depends on AR model order\n');

fprintf('\nCONCLUSION:\n');

fprintf('Welch method gives a stable and smooth spectral estimate.\n');

fprintf('AR method provides sharper spectral characteristics.\n');

fprintf('Both methods can be used to analyze the OFDM spectrum.\n');

Output