Multirate Signal Processing

8.1 Implementation and Analysis of Decimation and Interpolation in Multirate Signal Processing Using MATLAB

Program (MATLAB Code)

clc;

clear;

close all;

% Original Sequence

n = 0:40;

x = sin(0.3*pi*n);

% Decimation by 2

y = x(1:2:end);

nd = 0:length(y)-1;

% Interpolation by 2

v = zeros(1,2*length(x));

v(1:2:end) = x;

ni = 0:length(v)-1;

% Figure 1: Original Sequence

figure;

stem(n,x,'filled');

grid on;

title('Original Sequence x[n]');

xlabel('n');

ylabel('Amplitude');

% Figure 2: Decimated Sequence

figure;

stem(nd,y,'filled');

grid on;

title('Decimated Sequence by 2');

xlabel('n');

ylabel('Amplitude');

% Figure 3: Interpolated Sequence

figure;

stem(ni,v,'filled');

grid on;

title('Interpolated Sequence by 2');

xlabel('n');

ylabel('Amplitude');

% Figure 4: Comparison

figure;

stem(n,x,'b'); hold on;

stem(0:2:length(v)-2,v(1:2:end),'r');

legend('Original x[n]','Interpolated Samples');

grid on;

title('Comparison of Sequences');

xlabel('n');

ylabel('Amplitude');

Output

Fig 1

Fig 2

Fig 3

Fig 4

Result

1. A discrete-time sinusoidal sequence was successfully generated for multirate processing. 2. The discrete-time sequence was successfully decimated by a factor of 2, reducing the number of samples and the sampling rate.

3. The discrete-time sequence was successfully interpolated by a factor of 2, increasing the number of samples and the sampling rate.

4.The effects of down sampling and up sampling were successfully analysed by comparing the original, decimated, and interpolated signals.