Multirate Signal Processing

8.2 Polyphase Implementation of Decimation and Interpolation Filters Using MATLAB

Program (MATLAB Code)

clc;

clear;

close all;

%% Input Signal

Fs = 1000;                    % Sampling Frequency

t = 0:1/Fs:1;

% Test Signal

%

Time Vector

x = sin(2*pi*50*t) + 0.5*sin(2*pi*120*t);

%% FIR Low-Pass Filter Design

N = 20;                       % Filter Order

Fc = 0.4;

% Normalized Cutoff Frequency

h = fir1(N,Fc,hamming(N+1));

%% Decimation Factor

M = 2;

% Polyphase Decimation

y_dec = upfirdn(x,h,1,M);

%% Interpolation Factor

L = 2;

% Polyphase Interpolation

y_int = upfirdn(y_dec,h,L,1);

%% Adjust Length for Comparison

if length(y_int) > length(x)

y_int = y_int(1:length(x));

elseif length(y_int) < length(x)

x = x(1:length(y_int));

end

%% Time Axes

t1 = (0:length(x)-1)/Fs;

t2 = (0:length(y_dec)-1)/(Fs/M);

t3 = (0:length(y_int)-1)/Fs;

%% BEST GRAPH OUTPUT

figure('Name','Polyphase Decimation and Interpolation','NumberTitle','off');

% Original Signal

subplot(4,1,1)

plot(t1,x,'b','LineWidth',1.5)

title('Original Input Signal')

xlabel('Time (s)')

ylabel('Amplitude')

grid on

% Decimated Signal

subplot(4,1,2)

stem(t2,y_dec,'r','filled')

title('Decimated Signal (M = 2)')

xlabel('Time (s)')

ylabel('Amplitude')

grid on

% Interpolated Signal

subplot(4,1,3)

plot(t3,y_int,'k','LineWidth',1.5)

title('Interpolated/Reconstructed Signal (L = 2)')

xlabel('Time (s)')

ylabel('Amplitude')

grid on

% Comparison

subplot(4,1,4)

plot(t1,x,'b','LineWidth',1.5)

hold on

plot(t3,y_int,'r--','LineWidth',1.5)

title('Comparison of Original and Reconstructed Signals')

xlabel('Time (s)')

ylabel('Amplitude')

legend('Original Signal','Reconstructed Signal')

grid on

%% Display Information

disp('--------------------------------------');

disp('POLYPHASE DECIMATION & INTERPOLATION');

disp('--------------------------------------');

fprintf('Original Signal Length

fprintf('Decimated Signal Length

= %d\n',length(x));

= %d\n',length(y_dec));

fprintf('Interpolated Signal Length  = %d\n',length(y_int));

%% Separate Figure for Better Visualization

figure;

plot(t1,x,'b','LineWidth',2)

hold on

plot(t3,y_int,'r--','LineWidth',2)

title('Original vs Reconstructed Signal')

xlabel('Time (s)')

ylabel('Amplitude')

legend('Original Signal','Reconstructed Signal')

grid on

Output

--------------------------------------
POLYPHASE DECIMATION & INTERPOLATION
--------------------------------------
Original Signal Length      = 1001
Decimated Signal Length     = 511
Interpolated Signal Length  = 100

Fig 1

Fig 2

Result

1. The input signal was successfully decimated by a factor of 2 using a polyphase FIR decimation filter.

2.The input signal was successfully interpolated by a factor of 2 using a polyphase FIR interpolation filter.

3. The original signal was reconstructed from the decimated signal and closely matched the input signal.

4.The polyphase filter structure reduced computational complexity and efficiently performed multirate signal processing operations.